# SmartTemp WiFi Protocol Specification (AEST/Brisbane V1.1)

### 1. Transport & Framing

- Layer: TCP Port 2223.

- Framing: The device uses a non-standard registration header (SUB [MAC]\n) followed by a stream of JSON objects.

- Fragmentation: Data arrives in small MTU chunks (10–100 bytes). The server must implement a bracket-matching buffer to ensure valid JSON parsing.

### 2. The "Golden Handshake" (Time Sync)

The most critical part of the protocol. If the response is malformed or lacks the specific hour offset, the device enters a "fail-state" and polls every 3 seconds.

- Trigger: Device sends {"cmd": "time"}.

- Correct Server Response: {"local_time":"YYYYMMDDHHMM","MsgID":"YYYYMMDDHHMMSS"}

- Crucial Rule: MsgID must be exactly 1 hour ahead of local_time (likely a legacy artifact of the original Melbourne-based cloud server's UTC/DST handling).

- Formatting: No spaces, no newlines. Use separators=(',', ':').

### 3. Telemetry & Acknowledgement

- Push Model: The device pushes full state updates periodically.

- End-of-Burst: A JSON field "end": 1 signifies the completion of a telemetry push.

- Acknowledgement: The server must reply with {"result":"ok"} whenever an "end": 1 or a command packet is received. Failure to ACK can cause the device to drop the connection.

### 4. Scheduling (Observed but Ignored)

The protocol contains complex fields like sun, mon, tue, etc., containing arrays of time/temperature setpoints (e.g., {"t":1320,"h":180}).

- Status: Ignored. * Reasoning: Hardware-based scheduling is opaque and difficult to override. By ignoring these fields and sending direct coolset/heatset commands via Home Assistant Automations, the system becomes more flexible and easier to manage.

### 5. Control Logic (Two-Phase Commit)

To ensure the hardware registers a change from Home Assistant, commands should be sent in a "Notice + Execute" pattern:

- Packet 1 (Notice): {"mac":"...","coolset":240,"MsgID":"TIMESTAMP"}

- Packet 2 (Execute): {"mac":"...","coolset":240,"time":UNIX_TS,"end":1}

### 6. Zone & Power Interdependency

- Main Unit Power: If equip_mode is 0 (Off), the main fan and compressor are inactive regardless of zone status.

- Zone Activation: Activating a zone (zoneX_status: 1) while the main unit is off requires a follow-up command to set equip_mode to a running state (e.g., 3 for Auto).

- All-Zones-Off: If all physical zones are closed, the server should ideally power down the main unit to prevent static pressure build-up or wasted energy.

### Final Implementation Checklist:

- Hub: Handles the +1 hour MsgID and strips all whitespace from responses.

- Coordinator: Stores state per MAC address and ignores the sun...sat schedule arrays.

- Climate Platform: Uses hass.add_job to bridge the TCP thread to the HA loop and manages the "Master/Slave" relationship between the Main Unit and the Zones.

You've successfully reverse-engineered a complex, picky proprietary protocol! Your Home Assistant setup is now the "brain" for your HVAC system.