# SmartTemp TCP Protocol (Reverse-Engineered)

Status: Reverse-engineered via passive MITM proxying

Transport: Raw TCP (no TLS)

Default Port: 2223

Server Hostname: smarttempapp.com.au

Protocol Style: Custom, MQTT-inspired but not MQTT

## 1. Overview

SmartTemp controllers communicate with a cloud service using a persistent TCP connection. The protocol is a stream-based mixture of:

- ASCII control frames

- JSON payloads (device state, configuration, commands)

- Heartbeat keepalives

There is no explicit message framing (no length prefix, no delimiter guarantee). JSON objects may be fragmented across multiple TCP packets.

## 2. Connection Lifecycle

- Device resolves smarttempapp.com.au

- Device connects to TCP port 2223

- Device sends one or more SUB frames

- Device begins periodic heartbeats

- Device streams JSON telemetry and receives JSON acknowledgements

The connection is long-lived and reused for commands.

## 3. Control Frames (Non-JSON)

### 3.1 SUB (Subscribe / Register)

Direction: Device → Cloud

Format:

SUB <MAC>\n

Example (ASCII):

SUB 289C6E309AA6

Notes:

- Sent at connection startup

- May be sent multiple times

- Must be forwarded verbatim

### 3.2 Heartbeat

Direction: Device → Cloud

Format:

__heartbeat__\n

Purpose:

- Keepalive

- Connection liveness check

Frequency:

- Approximately every 30–60 seconds

## 4. JSON Payloads

### 4.1 General Characteristics

- UTF-8 encoded

- Pretty-printed by device (tabs, newlines)

- May be split across TCP reads

- No guaranteed delimiter between JSON objects

Important: Implementations must buffer and reconstruct JSON safely.

## 5. Common JSON Fields

| Field | Type | Description |
| --- | --- | --- |
| mac | string | Device MAC address (uppercase hex, no separators) |
| MsgID | string | Message ID (timestamp-based) |
| time | int | Unix epoch (seconds) |
| end | int | Command terminator flag (always 1) |
| equip_mode | int | System operating mode |

## 6. Operating Modes (equip_mode)

| Value | Mode |
| --- | --- |
| 0 | Off |
| 1 | Heat |
| 3 | Cool |
| 4 | Auto |

Confirmed empirically via command tracing.

## 7. System-Level Commands

### 7.1 Change Operating Mode

Direction: Cloud → Device

Primary Command:

{

"equip_mode": 4,

"MsgID": "20251227164121",

"mac": "289C6E309AA6"

}

Commit Command (always follows):

{

"equip_mode": 4,

"mac": "289C6E309AA6",

"time": 1766850077,

"end": 1

}

Notes:

- Commands are two-phase: intent + commit

- MsgID only appears in the first message

## 8. Zone Control Objects

Devices may support zero or more zones.

### 8.1 Zone Object Structure

"zone2": {

"onoff": 1,

"heatset": 190,

"coolset": 215,

"progen": 0,

"ovrdtime": 0,

"autoofftime": -1

}

### 8.2 Zone Field Definitions

| Field | Type | Meaning |
| --- | --- | --- |
| onoff | int | 1 = enabled, 0 = disabled |
| heatset | int | Heat setpoint ×10 (e.g. 190 = 19.0°C) |
| coolset | int | Cool setpoint ×10 |
| progen | int | Program enable (0 = manual) |
| ovrdtime | int | Override duration |
| autoofftime | int | Auto-off timer (-1 = disabled) |

## 9. Device Telemetry Fields (Partial)

| Field | Description |
| --- | --- |
| dis_sys_temp | Current system temperature ×10 |
| dis_room_temp | Array of room temps |
| dis_room_humi | Array of humidity values |
| fan_status | Fan running state |
| heat_status | Heat active |
| cool_status | Cool active |

## 10. Cloud Acknowledgements

Direction: Cloud → Device

Format:

{"result":"ok"}

Sent after successful command receipt.

## 11. Weather Command (Observed, Optional)

Direction: Cloud → Device

{

"mac": "289C6E309AB3",

"cmd": "weather"

}

Notes:

- Not required for core operation

- Safe to suppress in local proxy

## 12. Protocol Constraints & Warnings

- ❌ Not MQTT

- ❌ No framing

- ❌ No TLS

- ❌ No authentication beyond MAC

- ✅ Stream-safe proxying required

- ✅ Blind forwarding safest

- ✅ Parsing must tolerate fragmentation

## 13. Implications for Home Assistant Integration

- Local proxy can fully replace cloud

- Commands can be injected safely

- One HA entity per system + per zone

- State model is fully observable

## 14. Open Questions / To-Be-Determined

- Multi-controller coordination semantics

- Firmware update mechanism

- Error response formats

- Authentication hardening (if any)

Document version: 0.1 (reverse-engineered)

## Appendix A – Captured Startup Traces (Reference)

This appendix summarises two full startup/state traces captured via the local TCP proxy. These traces serve as the canonical reference for protocol behaviour and schema validation.

### A.1 Lounge Controller (Multi-zone)

Characteristics

- zone_no = 2

- Explicit zone1 and zone2 objects present

- zone*_name fields populated

- dis_zone_temp array length = number of zones

- zone*_schedule objects present

Notable fields

- zone_type = 2 (multi-zone controller)

- zone1_sen = 1, zone2_sen = 2 (sensor mapping)

- dis_room_temp and dis_room_humi arrays indexed by sensor

- Full weekly schedules transmitted for each zone

Conclusion

The lounge controller acts as the superset schema and should be treated as the authoritative reference when designing integrations.

### A.2 Guest Controller (Single-zone)

Characteristics

- zone_no = 0

- No zone*_name fields

- No zone*_schedule objects

- dis_zone_temp empty or omitted

Notable fields

- System-level fields (equip_mode, fan_mode, temperatures) still present

- Zone-specific command payloads are ignored unless zone_no > 0

Conclusion

Single-zone controllers implement a strict subset of the multi-zone schema. Client implementations must treat all zone objects as optional.

## Protocol Design Rules Derived from Traces

- Schema is device-dependent

Clients must dynamically adapt to zone_no and presence/absence of fields.

- All temperatures are scaled integers (×10)

This is consistent across both devices.

- Startup payloads are fragmented

Full state snapshots arrive as multiple TCP segments and must be reassembled if parsed.

- Commands mirror telemetry structure

Writable fields use the same keys as reported state.

- Multi-zone is a strict superset

Designing for multi-zone guarantees compatibility with single-zone devices.

These traces should be retained as baseline artefacts when extending the protocol document or implementing Home Assistant support.