| Type |  | Field | subfield | Description | Logic | Example |
| --- | --- | --- | --- | --- | --- | --- |
| Status Date - provided in Pair-key JSON |  |  |  |  |  |  |
|  | Pair-key Status Data |  |  |  |  |  |
|  |  | zone_no: |  | Number of Zones on this controller | 0 = no zones, n = number of zones | 2 |
|  |  | temp_max: |  | Maximum Temperature | Integer (Value / 10 = °C) | 320, |
|  |  | temp_min: |  | Minumum Temperature | Integer (Value / 10 = °C) | 180, |
|  |  | fan_status: |  | Fan speed Status | 0: Auto, 1: Low, 2: Medium, 3: High | 0, |
|  |  | heat_status: |  | Heating status | 0: off, 1:on | 0, |
|  |  | cool_status: |  | Cooling status | 0: off, 1:on | 0, |
|  |  | dis_room_temp: |  | Room Temp in non-zoned controller | Array of integers (Value / 10 = °C) | [244, 0, 0, 0], |
|  |  | dis_room_humi: |  | Room Humidity | Array of integers (Value = %) | [68, 0, 0, 0], |
|  |  | dis_zone_temp: |  | Zone Temp in zoned controller | Array of integers (Value / 10 = °C) | [239, 240], |
|  |  | equip_mode: |  | System Power/Mode | 0: Off, 1: Heat, 2: Cool, 4: Auto | 0, |
|  |  | fan_mode: |  | Fan is Continious or Automatic | 0: Auto, 1: Coninious | 0, |
|  |  | fan_speed: |  | Fan Velocity | 0: Auto, 1: Low, 2: Medium, 3: High | 1, |
|  |  | mac: |  | mac address of controller |  | 289C6E309AB3, |
|  | additional Non-zoned Pair-key data |  |  |  |  |  |
|  |  | sys_set: | { |  |  |  |
|  |  |  | heatset: | Target Heat Setpoints | Integer (Value / 10 = °C) | 200, |
|  |  |  | coolset: | Target Cool Setpoints | Integer (Value / 10 = °C) | 220, |
|  |  |  | progen: | Program enable (0 = manual) |  | 0, |
|  |  |  | ovrdtime: | Override duration |  | 0, |
|  |  |  | autoofftime: | Time to turn off Controller (-1 = Disabled) | same format as time | 1766850077 |
|  |  | }, |  |  |  |  |
|  | additional Zoned Pair-key Status Data |  |  |  |  |  |
|  |  | zone[n]_name: |  | If Zoned: Name of Zone [n] | Text | Master Bedroom, |
|  |  | zone[n]: | { | Zone [n] settings |  |  |
|  |  |  | onoff: | Zone on/off |  | 1, |
|  |  |  | heatset: | Target Heat Setpoints | Integer (Value / 10 = °C) | 195, |
|  |  |  | coolset: | Target Cool Setpoints | Integer (Value / 10 = °C) | 255, |
|  |  |  | progen: | Program enable (0 = manual) |  | 0, |
|  |  |  | ovrdtime: | Override duration |  | 0, |
|  |  |  | autoofftime: | Time to turn off Controller (-1 = Disabled) | same format as time | -1 |
|  |  | }, |  |  |  |  |